ANTENNAS

Antennas

•	a device that provides a transition between guided electromagnetic waves in wires and electromagnetic waves in free space.

Reciprocity

•	Antennas can usually handle this transition in both directions (transmitting and receiving EM waves).

This property is called reciprocity.

•	The antenna’s size and shape largely determines the frequencies it can handle and how it radiates electromagnetic waves.

Antenna polarization

•	The polarization of an antenna refers to the orientation of the electric field it produces.

o Polarization is important because the receiving antenna should have the same polarization as the transmitting antenna to maximize received power.

•	Types of Polarization

o Horizontal Polarization

o Vertical Polarization

o Circular Polarization

   Electric and magnetic field rotate at the frequency of the transmitter

   Used when the orientation of the receiving antenna is unknown

   Will work for both vertical and horizontal antennas

   Right Hand Circular Polarization (RHCP)

   Left Hand Circular Polarization (LHCP)

   Both antennas must be the same orientation (RHCP or LHCP)

Wavelength and antennas

•	recall that wavelength (λ) and frequency (f ) of an electromagnetic wave in free space are related by the speed of light (c). (c=3.0x108 m/s )

•	The dimensions of an antenna are usually expressed in terms of wavelength (λ).

o Low frequencies imply long wavelengths, hence low frequency antennas are very large.

o High frequencies imply short wavelengths, hence high frequency antennas are usually small.

What to look for in Antennas?

•	Frequency/Wavelength

•	Beam Pattern

•	Bandwidth

•	Gain

Basic Antenna

•	An antenna can be a length of wire, a metal rod, or a piece of metal tubing.

•	Antennas radiate most effectively when their length is directly related to the wavelength of the transmitted signal.

•	Most antennas have a length that is some fraction of a wavelength.

•	One-half and one-quarter wavelengths are most common.

•	isotropic point source - Power from an isotropic point source is equally distributed in all directions. It is completely unfocused. Antenna radiates equally in all directions.

Antenna gain (G)

•	Because an antenna is a passive device, the power radiated cannot be greater than the input power.

•	The ability of an antenna to focus electro-magnetic energy is defined by its gain.

•	Antenna gain is expressed as a ratio of the effective radiated output power (Pout ) to the input power (Pin )

•	The gain of an antenna is a measure of power transmitted relative to that transmitted by an isotropic source.

•	Antenna gain relative to an isotropic source is expressed in decibels as dBi.

Effective Radiated Power

•	The effective radiated power (ERP or EIRP) is the gain of an antenna (with respect to an isotropic radiator) multiplied by its input power.

EIRP = Input Power x Antenna Gain

•	For example, a highly directional antenna with a gain of 7 has an input power of 1-kW.  Its ERP is therefore 7 kW.

Dipole Antenna

•	One of the most widely used antenna types is the half-wave dipole.

•	The half-wave dipole, also called a doublet, is formally known as the Hertz antenna.

•	A dipole antenna is two pieces of wire, rod, or tubing that are one-quarter wavelength long at the operating resonant frequency.

Electric and Magnetic Fields around a Transmission Line

Converting a Transmission Line into an Antenna

The Dipole Antenna

Three-dimensional radiation pattern for a dipole

Antenna Losses

•	Radiation Loss

o Caused by radiation resistance

o Results in radiated RF energy

•	Resistive Loss

o Results in heat due to resistance of conductor

Dipole Definitions

•	The dipole has an impedance of 73 Ω at its center, which is the radiation resistance.

o An antenna ideally appears as a resistor to the transmitter. This “radiation resistance” does not dissipate power in the form of heat; the power is dissipated as radiated electromagnetic energy.

•	An antenna is a frequency-sensitive device, and a particular antenna can be operated over a range of frequencies (BW).

•	At the resonant frequency, the antenna appears to be a pure resistance of 73 Ω

Dipole Antenna gain (G)

•	A dipole antenna gain is 1.64

•	A half-wave dipole antenna has a power gain of 1.64 (or 2.15 dB) over an isotropic source.

•	Antenna gain relative to a dipole antenna can be expressed in decibels as dBd.

•	Thus, an antenna with a gain of 3 dBd would have a gain of 5.15 dBi (3 dB + 2.15 dB)

Dipole Antenna Lengths

•	A dipole resonates best when it is approx. 95% of the actual “half-wavelength length”

•	Shortcut:

Lfeet = 468/f MHz  (This is in Feet)

•	1 ft = .3048 m

•	Dipole hung vertically is closest to an isotropic radiator

•	Bottom of dipole antenna should be at least ½ a wavelength off the ground

o May make total structure height unreasonable

Conical Antenna

•	A common way to increase bandwidth is to use a version of the dipole antenna known as the conical antenna.

•	The center radiation resistance of a conical antenna is much higher than the 73 Ω usually found when straight-wire or tubing conductors are used.

•	The primary advantage of conical antennas is their tremendous bandwidth.

•	They can maintain a constant impedance and gain over a 4:1 frequency range.

Antenna Beam Width

Marconi or Ground-Plane Vertical Antenna

•	The quarter-wavelength vertical antenna, also called a

Marconi antenna is widely used.

•	It is similar in operation to a vertically mounted dipole antenna.

•	The Marconi antenna is half the length of a dipole antenna.

•	The earth acts as a type of electrical “mirror,” effectively providing the other quarter wavelength making it equivalent to a vertical dipole.

Marconi Advantages

•	Half the length of a dipole

•	Can be located at earth level without degrading performance

•	Has omni-directional radiation pattern similar to dipole

Marconi Disadvantages

•	Gain is slightly lower than a dipole (about 1 dB less), but for our purposes we will consider them the same

•	Antenna is extremely dependent on conductivity of the earth

•	Using a counterpoise will improve conductivity

Counterpoise

•	Sometimes connecting a monopole antenna to the ground is not feasible. Create a ground.

o Antennas mounted on buildings or towers

o Soil is highly resistive (dry)

•	A counterpoise is a flat structure of wire or screen that forms an artificial

reflecting surface for the monopole antenna if the actual earth cannot be used

•	Counterpoise requirements

o Must be at least equal to or larger than the antenna.

o Should extend in equal distances from the antenna.

o Must be insulated from the ground.

•	The performance of a quarter-wave antenna (either well-grounded or using a counterpoise) is essentially the same as a half-wave dipole antenna.

•	“Drooping” radials is one way to adjust antenna impedance.